Design Tool

1. System Design Overview

The system is designed as a TinyML-based cabin condition monitoring device. It uses sensor readings from the Arduino to classify the cabin state into three classes: Normal, Caution, and Alert. The complete system follows a simple pipeline: sensor reading, preprocessing, window creation, model inference, class decision, and LCD display. The model is trained in Google Colab, then converted into Arduino header files so the final prediction can run locally on the Arduino without needing a laptop or internet connection.

2. Hardware Design

The hardware design is based on the Arduino Nano 33 BLE Sense, which is used as the main controller. The system uses a thermistor for temperature sensing, an LDR for light sensing, and the onboard IMU accelerometer for motion sensing. The LCD is connected through I2C and is used to display the predicted class. The thermistor and LDR provide environmental information about cabin heat and lighting, while the accelerometer helps detect movement intensity. Together, these sensors provide the input needed to classify the cabin condition.

3. Data Collection Design

The data collection design was created to collect labeled samples for the three classes. The Arduino reads temperature, LDR, and acceleration values, then prints them in CSV format with the selected label. The user selects the class using serial commands: Normal, Caution, or Alert. After starting the collection, the system skips the first few warmup readings to avoid unstable sensor values, then collects 1000 valid samples for each class. The final dataset contains samples , with 1000 samples per class, making the dataset balanced.

4. Feature Design

The final training features are temperature, LDR, and movementScore. Temperature represents cabin heating, LDR represents light intensity, and movementScore represents the strength of motion. Instead of using raw accX, accY, and accZ directly, movementScore is calculated from the change in acceleration between consecutive readings. This makes the model focus on movement intensity rather than the exact direction of the Arduino board. It also reduces the number of input features, making the model smaller and more suitable for Arduino deployment.

5. Preprocessing Design

The preprocessing stage converts the raw CSV data into a clean training table. The code detects the required columns, extracts temperature and LDR values, creates movementScore if needed, and keeps only the useful columns: temperature, LDR, movementScore, and label. The features are normalized using StandardScaler so that all values are placed on a balanced scale. This is important because temperature, LDR, and movementScore have different numeric ranges. The same scaling values are saved later in scaler.h so that Arduino can preprocess live sensor readings in the same way as Colab.

6. Windowing Design

The model does not classify one sensor row at a time. Instead, it uses sliding windows. Each window contains 10 consecutive sensor readings, and each reading has 3 features: temperature, LDR, and movementScore. The label of the window is taken from the newest row in that window. This allows the CNN to learn short-time behavior instead of relying on a single instant. Since the sampling interval is 300 ms, each 10-sample window represents about 3 seconds of cabin behavior.

7. CNN Model Design

The model is designed as a small CNN suitable for TinyML deployment. The input shape is one window of 10 readings, 3 features, and 1 channel. The CNN uses a Conv2D layer with 4 filters to learn short-time patterns from the sensor window. After that, the output is reshaped into a flat vector, passed through a dense layer, and finally passed to a softmax output layer with 3 outputs. The output classes are Normal, Caution, and Alert. The model is intentionally small to reduce memory usage and make real-time inference possible on the Arduino.

8. Model Conversion Design

After training in Colab, the Keras model is converted into a TensorFlow Lite model. This TFLite model is then converted into model.h, which stores the trained model as a C array. The scaler values are saved into scaler.h, and the class names are saved into labels.h. These files are included in the Arduino inference code. This design allows the Arduino to run the trained model locally using TensorFlow Lite Micro.

9. Inference Design

During inference, the Arduino continuously reads live sensor values. It calculates movementScore from the change in accelerometer readings, stores the latest 10 readings in a window buffer, normalizes the values using the means and scales from scaler.h, and places the window into the model input tensor. The TensorFlow Lite Micro interpreter then runs the model using Invoke(). The model outputs three probabilities, one for each class. The class with the highest probability is selected as the final prediction.

10. LCD Output Design

The LCD is used as the user interface of the system. After the model predicts the cabin condition, the Arduino displays the class name on the LCD. The output can be Normal, Caution, or Alert. This makes the system easy to understand during the live demo because the user can directly see the real-time classification result without needing to open the Serial Monitor.

11. Design Constraints

The main design constraint is the limited memory and processing power of the Arduino. Because of this, the model must be lightweight, the number of features must be limited, and the TensorFlow Lite tensor arena must be carefully selected. The model size is about 6.4 KB, and the tensor arena is set to 48 KB. Another important constraint is that the preprocessing on Arduino must exactly match the preprocessing used in Colab. If the scaling or feature order changes, the model may give wrong predictions.

12. Design Justification

This design was selected because it balances accuracy, simplicity, and Arduino compatibility. Using temperature, LDR, and movementScore gives enough information to distinguish between Normal, Caution, and Alert conditions while keeping the input small. The sliding window allows the CNN to learn short-time sensor patterns. The small CNN architecture reduces memory usage and avoids unnecessary complexity. Finally, converting the model into Arduino header files allows the complete system to run locally on the microcontroller, which is the main goal of a TinyML project.